SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1 - CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA1208 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1 - Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| Student Name: | GOPIKA P | Reg.No.: | 192412306 |
| Department: | ECE | Date: | 2/09/2026 |

CO5 Statement:

Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt.

ANNEXURE A

Constraint-Based Problem Solving

1. Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

• CPU utilization must be below 15%

• Multiple sensors generate interrupts every 1 ms

• Use vectored interrupt handling

• Select between memory-mapped and I/O-mapped I/O for optimal latency

2. Design an I/O subsystem under these constraints:

• High-speed storage requires throughput > 3 GB/s

• Must use NVMe over PCIe

• CPU should not handle bulk data transfer

• Integrate DMA controller with interrupt notification

3. Construct a multi-device communication architecture with constraints:

• 6 peripherals using USB, 2 high-speed devices using Thunderbolt

• Avoid interrupt starvation

• Use hardware and software interrupts

• Ensure fair bus arbitration

4. Design a data acquisition system with constraints:

• Continuous data stream at 500 MB/s

• Minimal CPU intervention required

• Must compare DMA transfer vs interrupt-driven I/O

• Use memory-mapped I/O for device registers

5. Construct an interrupt handling mechanism with constraints:

• Support nested interrupts

• Maximum interrupt latency < 10 µs

• Use vectored interrupts for high-priority devices

• Use software interrupts for background tasks

ANSWERS – CONSTRAINT-BASED PROBLEM SOLVING

QUESTION 1

Construct an I/O communication mechanism for a real-time embedded system with the following constraints: CPU utilization below 15%, multiple sensors generating interrupts every 1 ms, vectored interrupt handling, and a choice between memory-mapped and I/O-mapped I/O for optimal latency.

1.1 Problem Understanding

The system is a real-time embedded platform connected to multiple sensors. Every sensor can generate an interrupt at a period of 1 ms. Therefore, the design must respond quickly and predictably while keeping processor utilization below 15%. The interrupt mechanism must be vectored so that each interrupt source can reach its appropriate service routine without a long software search.

Real-time response is the primary requirement because sensor events occur at a fixed 1 ms interval.

CPU utilization must remain below 15%, so interrupt service routines must be short and most data movement should be handled by hardware.

Vectored interrupts reduce dispatch overhead by providing a direct vector to the corresponding ISR.

Device registers should be accessed using memory-mapped I/O because it gives the processor a uniform address space and supports efficient load/store operations.

Interrupt priority should be assigned so that safety-critical or high-rate sensors are serviced before lower-priority sources.

1.2 Proposed I/O Architecture

The proposed architecture contains sensors, interrupt lines, a programmable interrupt controller, a CPU, memory-mapped device registers and a shared memory buffer. Each sensor has a dedicated interrupt vector. The ISR reads only the minimum status/data required, places the event into a circular buffer and clears the interrupt. A background task processes the buffered data.

| Block | Function |
| --- | --- |
| Sensors | Generate measurements and interrupt requests every 1 ms. |
| Interrupt Controller | Accept, prioritize and vector interrupt requests. |
| CPU | Executes short ISRs and real-time control tasks. |
| Memory-Mapped Registers | Provide low-overhead access to sensor control/status/data registers. |
| Circular Buffer | Temporarily stores sensor samples so the ISR remains short. |
| Background Task | Performs non-critical processing outside the ISR. |

1.3 Interrupt Operation

Sensor detects a new sample and asserts its interrupt request.

The interrupt controller identifies the source and selects its vector.

CPU saves the minimum context and jumps directly to the corresponding ISR.

The ISR reads the sensor register and stores the sample in a preallocated circular buffer.

The ISR acknowledges the interrupt and returns immediately.

A background task consumes buffered samples and performs filtering, logging or communication.

1.4 CPU Utilization Calculation

If there are N sensors, each generating one interrupt every 1 ms, the total interrupt rate is N × 1000 interrupts/s. Let C be the average CPU time used by one ISR in seconds. The approximate interrupt CPU utilization is U = N × 1000 × C × 100%.

| Number of sensors | ISR time per interrupt | Interrupt CPU utilization |
| --- | --- | --- |
| 4 | 20 µs | 8.0% |
| 6 | 20 µs | 12.0% |
| 8 | 15 µs | 12.0% |
| 10 | 10 µs | 10.0% |

For example, with six sensors and a 20 µs ISR, utilization is 6 × 1000 × 20×10⁻⁶ ×100 = 12%, leaving approximately 3 percentage points of the 15% limit for other interrupt-related overhead. Therefore, the ISR must remain short and deterministic.

1.5 Memory-Mapped vs I/O-Mapped I/O

| Criterion | Memory-Mapped I/O | I/O-Mapped I/O |
| --- | --- | --- |
| Addressing | Uses normal memory address space | Uses separate I/O address space |
| Instructions | Normal load/store instructions | Special I/O instructions may be required |
| Programming | Uniform and simple | Separate I/O operations |
| Latency | Generally efficient on modern processors | Can be efficient but architecture dependent |
| Recommendation | Selected | Not selected |

Memory-mapped I/O is selected because the requirement emphasizes optimal latency in a real-time embedded system. Device registers can be accessed with normal load/store instructions, and the address space integrates naturally with the processor's memory system. The design should reserve non-cacheable/strongly ordered attributes for device registers where required by the processor architecture.

1.6 Final Solution and Constraint Check

| Constraint | Design decision | Status |
| --- | --- | --- |
| CPU utilization <15% | Short vectored ISR + circular buffer; target 12% in representative case | Satisfied |
| Interrupt every 1 ms | Dedicated interrupt vectors and priority controller | Satisfied |
| Vectored interrupt handling | One vector/handler entry per interrupt source | Satisfied |
| Optimal I/O latency | Memory-mapped I/O for device registers | Selected |
| Real-time behavior | Preallocated buffers and bounded ISR execution | Satisfied |

Hence, the recommended solution is a memory-mapped, vectored interrupt architecture with prioritized sensor interrupts, short ISRs and deferred background processing. This minimizes CPU overhead while preserving deterministic response.

QUESTION 2

Design an I/O subsystem under the constraints: high-speed storage throughput greater than 3 GB/s, NVMe over PCIe, CPU should not handle bulk transfer, and DMA must be integrated with interrupt notification.

2.1 Problem Understanding

The system requires a high-performance storage path. A throughput above 3 GB/s is beyond what is appropriate for CPU-driven programmed I/O for every data word or small block. NVMe over PCIe is therefore used as the storage interface, while DMA moves data between the NVMe controller and system memory without continuous CPU intervention.

2.2 Proposed Architecture

| Component | Role |
| --- | --- |
| NVMe SSD | High-speed non-volatile storage device. |
| PCIe Root Complex | Connects the host CPU/memory system to the NVMe controller. |
| NVMe Controller | Maintains submission/completion queues and performs storage commands. |
| DMA Engine | Moves data between device and system memory. |
| System RAM | Destination/source for bulk data buffers. |
| Interrupt/MSI-X mechanism | Notifies CPU when command completion or important events occur. |
| CPU | Builds commands, manages queues and handles completion; does not copy bulk data. |

2.3 Data Transfer Sequence

CPU allocates or selects aligned memory buffers.

CPU places an NVMe command in the submission queue.

NVMe controller fetches the command through PCIe.

The controller transfers data directly between the SSD and host memory using DMA.

After the transfer completes, the controller writes a completion entry.

An interrupt notification informs the CPU that the completion queue contains a completed request.

CPU processes completion metadata and submits the next batch of commands.

2.4 Throughput Analysis

The required throughput is greater than 3 GB/s. For example, at 3.2 GB/s, a one-second transfer moves approximately 3.2 billion bytes. The CPU cannot efficiently copy each byte or repeatedly poll device status. DMA removes the bulk-copy operation from the CPU path.

| Parameter | Representative design value |
| --- | --- |
| Required throughput | >3 GB/s |
| Selected interface | PCIe |
| Storage protocol | NVMe |
| Bulk transfer mechanism | DMA |
| Completion notification | Interrupt, preferably MSI-X |
| CPU role | Queue management and completion processing |

2.5 Why DMA Is Necessary

CPU is released from repetitive data-copy operations.

Memory bandwidth is used more efficiently because transfers are performed by hardware.

Large transfers can be grouped into descriptors or queue entries.

Interrupts can be generated on completion of batches instead of every small transfer.

NVMe queueing allows multiple outstanding commands and parallel device operation.

2.6 Constraint Verification

| Constraint | Solution | Result |
| --- | --- | --- |
| >3 GB/s throughput | NVMe SSD over PCIe with multiple outstanding commands | PASS |
| Must use NVMe over PCIe | PCIe-connected NVMe controller | PASS |
| CPU must not handle bulk data | DMA direct transfer to/from RAM | PASS |
| DMA + interrupt notification | DMA transfer completion followed by interrupt/MSI-X | PASS |

The final I/O subsystem is therefore a PCIe/NVMe/DMA architecture with queue-based asynchronous operation and interrupt-based completion notification. It satisfies the stated performance and CPU-intervention constraints.

QUESTION 3

Construct a multi-device communication architecture with constraints: six USB peripherals, two high-speed Thunderbolt devices, avoidance of interrupt starvation, hardware and software interrupts, and fair bus arbitration.

3.1 Problem Understanding

The system combines low-to-medium speed USB peripherals with two high-speed Thunderbolt devices. Because several devices may request service concurrently, a centralized priority policy alone could starve lower-priority devices. The design therefore uses hierarchical scheduling, bounded interrupt service and fair arbitration.

3.2 Proposed Communication Architecture

| Layer | Design |
| --- | --- |
| CPU / Memory | Runs device drivers, interrupt handlers and scheduling software. |
| Interrupt Controller | Handles hardware interrupt requests and priorities. |
| USB Host Controller | Manages six USB peripherals using queued transactions. |
| USB Hub | Provides sufficient ports and distributes the six peripherals. |
| Thunderbolt Controller | Connects two high-speed devices through dedicated high-speed paths. |
| Bus/Interconnect Arbiter | Uses weighted round-robin or time-sliced arbitration. |
| Software Interrupt Layer | Schedules deferred/background processing and driver work. |

3.3 Device Allocation

Six USB peripherals are connected through an appropriate USB host controller/hub arrangement. The two Thunderbolt devices are attached to high-speed Thunderbolt ports/controllers. High-rate Thunderbolt transfers are allowed larger data bursts, but an arbitration policy reserves service opportunities for USB traffic.

3.4 Interrupt Handling

Hardware interrupts are generated by controllers when urgent device events, completions or errors occur.

High-priority hardware interrupts are serviced immediately but their ISR duration is bounded.

Software interrupts or deferred procedure mechanisms handle non-critical driver work outside the hardware ISR.

Interrupt coalescing may be used for high-rate devices so that the CPU is not interrupted for every small packet.

Priority aging or a fairness mechanism prevents low-priority devices from waiting indefinitely.

3.5 Fair Bus Arbitration

Weighted round-robin arbitration is selected. Each active device class receives a service opportunity according to an assigned weight, but no class can hold the bus indefinitely. After a device consumes its quota, the arbiter advances to the next eligible requester.

| Requester | Example weight | Purpose |
| --- | --- | --- |
| Thunderbolt device 1 | 3 | High bandwidth |
| Thunderbolt device 2 | 3 | High bandwidth |
| USB group | 2 | Fair service for six peripherals |
| Low-priority maintenance | 1 | Background service |

The exact weights are implementation parameters. The important constraint is that the scheduler has a finite service quantum and rotates among active requesters.

3.6 Avoiding Interrupt Starvation

Use priority levels but impose a maximum service time for each ISR.

Coalesce repetitive interrupts from high-speed devices.

Move lengthy processing to software interrupts/deferred handlers.

Use priority aging so a waiting USB request eventually gains service.

Keep a separate queue per device or device class.

Measure worst-case waiting time during simulation.

3.7 Constraint Verification

| Constraint | Design response | Status |
| --- | --- | --- |
| 6 USB peripherals | USB host controller/hub with six device connections | PASS |
| 2 Thunderbolt devices | Two high-speed Thunderbolt connections | PASS |
| Avoid interrupt starvation | Bounded ISR + aging + interrupt coalescing | PASS |
| Hardware and software interrupts | Controller IRQ + deferred software interrupt processing | PASS |
| Fair bus arbitration | Weighted round-robin/time slicing | PASS |

QUESTION 4

Design a data acquisition system with constraints: continuous data stream at 500 MB/s, minimal CPU intervention, comparison of DMA versus interrupt-driven I/O, and memory-mapped I/O for device registers.

4.1 Problem Understanding

The data acquisition system continuously receives 500 MB/s. At this rate, interrupting the CPU for every small data unit would create excessive overhead. DMA is therefore the preferred bulk-transfer mechanism. Memory-mapped I/O is used for the acquisition device's control, status and configuration registers.

4.2 Proposed Architecture

| Block | Function |
| --- | --- |
| Sensor/ADC Front End | Produces continuous sampled data. |
| Acquisition Controller | Buffers incoming samples and exposes registers. |
| DMA Controller | Transfers blocks directly into RAM. |
| Double/Ring Buffer | Allows acquisition and processing to overlap. |
| CPU | Configures DMA, processes completed blocks and handles exceptions. |
| Memory-Mapped Registers | Provide control/status access to the acquisition hardware. |
| Application | Consumes processed data from memory buffers. |

4.3 DMA Transfer Operation

CPU configures acquisition registers through memory-mapped I/O.

CPU programs DMA descriptors with source, destination, length and control information.

Acquisition controller receives samples continuously.

DMA transfers a block directly into RAM.

DMA generates an interrupt when the block or a group of blocks is complete.

CPU processes the completed buffer while DMA fills the next buffer.

The process repeats continuously using a ring or double-buffer structure.

4.4 DMA vs Interrupt-Driven I/O

| Criterion | DMA | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU intervention | Low | High |
| Suitable for 500 MB/s | Yes | No, if interrupt frequency is high |
| Bulk transfer | Hardware controlled | CPU participates repeatedly |
| Latency | Good for block transfer | Good for small events |
| CPU utilization | Low | Can become excessive |
| Implementation | Requires DMA setup | Simpler for low-rate devices |
| Selection | Preferred | Not preferred |

4.5 Quantitative Reasoning

At 500 MB/s, if the system generated one interrupt for every 4 KB block, the interrupt rate would be approximately 500,000,000 / 4096 ≈ 122,070 interrupts/s. Such a rate is unnecessarily high for a CPU-driven data path. With a 1 MB DMA block, the interrupt rate becomes approximately 500 interrupts/s, while each interrupt represents a substantial amount of transferred data.

| DMA block size | Approximate interrupts/s at 500 MB/s |
| --- | --- |
| 4 KB | 122,070 |
| 64 KB | 7,629 |
| 256 KB | 1,907 |
| 1 MB | 500 |
| 4 MB | 125 |

A moderate block size should be selected because very large blocks can increase buffering latency, while very small blocks increase interrupt frequency. A ring buffer can provide continuous operation.

4.6 Memory-Mapped I/O

Memory-mapped I/O is used for device registers such as START, STOP, STATUS, DMA_ADDRESS, DMA_LENGTH and INTERRUPT_STATUS. These addresses must be configured as device/non-cacheable memory according to the processor architecture so that reads and writes reach the hardware correctly.

4.7 Constraint Verification

| Constraint | Design decision | Status |
| --- | --- | --- |
| 500 MB/s continuous stream | DMA + ring buffer + high-bandwidth memory path | PASS |
| Minimal CPU intervention | CPU handles setup and block completions only | PASS |
| Compare DMA vs interrupt-driven I/O | Quantitative interrupt-rate comparison included | PASS |
| Memory-mapped device registers | All acquisition control/status registers mapped into memory space | PASS |

QUESTION 5

Construct an interrupt handling mechanism with constraints: support nested interrupts, maximum interrupt latency below 10 µs, vectored interrupts for high-priority devices, and software interrupts for background tasks.

5.1 Problem Understanding

The interrupt controller must support priority levels and nesting. A high-priority device must be able to preempt a lower-priority ISR when its interrupt arrives. The worst-case latency from interrupt assertion to start of the high-priority ISR must remain below 10 µs.

5.2 Proposed Interrupt Architecture

| Component | Function |
| --- | --- |
| Priority Interrupt Controller | Ranks pending interrupt requests. |
| Vector Table | Maps interrupt sources to their ISR entry points. |
| CPU Context Save | Preserves the preempted task/ISR state. |
| High-Priority ISR | Executes the shortest urgent action. |
| Nested Interrupt Logic | Allows a higher-priority interrupt to preempt a lower-priority ISR. |
| Software Interrupt/Deferred Handler | Performs background or lengthy processing. |
| Timer/Watchdog | Measures service time and detects abnormal delays. |

5.3 Nested Interrupt Sequence

A low-priority interrupt starts executing its ISR.

A higher-priority interrupt arrives.

The controller marks the higher-priority request as pending and permits preemption.

CPU saves the minimum required context.

The vector for the high-priority source is fetched.

The high-priority ISR executes its bounded critical operation.

The high-priority ISR acknowledges the interrupt and returns.

CPU restores the previous context and resumes the low-priority ISR.

5.4 Latency Budget

Interrupt latency is the time from interrupt assertion to execution of the first instruction of the required ISR. To satisfy the <10 µs requirement, the complete path must include controller recognition, CPU pipeline response, context saving, vector fetch and entry overhead.

| Latency component | Design budget |
| --- | --- |
| Interrupt controller recognition | 1.0 µs |
| CPU response/pipeline | 1.5 µs |
| Context save | 2.0 µs |
| Vector fetch/dispatch | 1.0 µs |
| Safety margin | 2.0 µs |
| Total representative budget | 7.5 µs |

The representative budget of 7.5 µs is below 10 µs, leaving 2.5 µs margin. The actual processor and interrupt-controller datasheets must be used for a real implementation; the table demonstrates the constraint-based design method.

5.5 Hardware vs Software Interrupts

| Interrupt type | Use | Priority |
| --- | --- | --- |
| Hardware interrupt | External/high-priority device events | High |
| Hardware timer interrupt | Real-time scheduling/timing | High |
| Software interrupt | Deferred background driver work | Low/medium |
| Exception | Processor-detected exceptional conditions | Architecture-defined |

High-priority hardware interrupts are kept short. Software interrupts are used to defer non-urgent work so that a long background operation cannot block time-critical hardware service.

5.6 Constraint Verification

| Constraint | Solution | Status |
| --- | --- | --- |
| Nested interrupts | Priority-based preemption and context save/restore | PASS |
| Maximum latency <10 µs | Representative bounded latency budget = 7.5 µs | PASS* |
| Vectored interrupts for high-priority devices | Dedicated vector entries | PASS |
| Software interrupts for background tasks | Deferred software interrupt mechanism | PASS |

*The 7.5 µs value is a design budget, not a measured processor result. For final laboratory validation, the latency must be measured using the selected processor/simulator.

OVERALL INTEGRATED ANALYSIS

The five designs apply the same constraint-based reasoning process: identify the hard performance limit, choose a hardware/software mechanism that directly addresses the bottleneck, quantify the expected load, and verify each requirement. The common principle is to keep CPU work focused on control, scheduling and exceptional conditions while hardware mechanisms perform repetitive high-volume operations.

Comparison of the Five Solutions

| Question | Main constraint | Selected mechanism | Primary reason |
| --- | --- | --- | --- |
| Q1 | CPU <15%, 1 ms sensor interrupts | Vectored memory-mapped I/O + short ISRs | Low latency and bounded CPU load |
| Q2 | >3 GB/s storage | NVMe over PCIe + DMA + interrupt notification | High throughput with low CPU copying |
| Q3 | 8 devices + fairness | USB/Thunderbolt + priority + weighted arbitration | Prevents starvation |
| Q4 | 500 MB/s continuous acquisition | DMA + ring buffer + memory-mapped registers | Minimal CPU intervention |
| Q5 | Latency <10 µs + nesting | Priority vectored nested interrupts | Fast high-priority response |

Critical Trade-Offs

Lower interrupt latency generally requires more hardware support and careful priority design.

Longer DMA blocks reduce interrupt frequency but can increase buffering latency.

High-priority interrupt service improves responsiveness but can delay lower-priority work; fairness mechanisms are therefore necessary.

Memory-mapped I/O simplifies access and can provide efficient load/store operations, but device regions must be correctly configured as non-cacheable or strongly ordered when required.

Interrupt coalescing reduces CPU overhead but can increase the time before software learns about individual events.

Nested interrupts improve urgent-event response but increase context-management complexity and require careful stack sizing.

Software interrupts improve system responsiveness by deferring long work, but the deferred queues must be sized so that backlog does not grow without bound.

FINAL CONCLUSION

The proposed architectures satisfy the constraints by matching each workload to an appropriate I/O communication technique. Q1 uses vectored memory-mapped I/O with short prioritized ISRs to keep CPU utilization below the limit. Q2 uses NVMe over PCIe with DMA and interrupt-based completion so that high-speed storage can exceed 3 GB/s without CPU-driven bulk copying. Q3 combines USB and Thunderbolt with bounded interrupt service, software-deferred processing and fair bus arbitration to avoid starvation. Q4 selects DMA over interrupt-driven bulk I/O for the 500 MB/s acquisition stream and uses memory-mapped registers for device control. Q5 implements nested priority interrupts with vectored high-priority handlers and software interrupts for background work, using a bounded latency budget below 10 µs.

Therefore, the overall engineering decision is to use hardware-assisted data movement and carefully bounded interrupt processing wherever data rates are high or timing constraints are strict. The final designs should be validated on the selected processor/controller platform using measured CPU utilization, throughput, interrupt rate, worst-case latency and fairness metrics.

REFERENCES

Computer Architecture for Multicore Systems, Course Code CSA12 – concepts on I/O communication, interrupt handling, DMA and bus interfaces.

Course assignment brief supplied by SIMATS Engineering, Assessment Tool 1 – Constraint-Based Problem Solving.

Standard processor and peripheral-controller documentation should be consulted for the exact interrupt-vector, DMA, PCIe, USB and memory-ordering behavior of the selected implementation platform.

Q5 CONTINUATION

• Use vectored interrupts for high-priority devices

• Use software interrupts for background tasks

Code of Conduct:

I  GOPIKA (192412306) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (25%) | Constraint Analysis (25%) | Solution Development (25%) | Application of Concepts (15%) | Justification (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing / Level 1 – Beginning |
| --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints / Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization / No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints / Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance / Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification / No justification provided |