1. PROBLEM UNDERSTANDING AND SYSTEM FORMULATION

1.1 Problem Statement

Urban traffic congestion causes increased travel time, fuel consumption, air pollution and delays in emergency services. Traditional traffic signals generally operate using fixed timings and may not respond effectively to changing traffic density.

The proposed Intelligent Traffic Management System (ITMS) uses traffic cameras, vehicle sensors and road-monitoring devices to collect real-time traffic information. The computer architecture processes this information and dynamically supports traffic signal decisions.

The system focuses on three major situations:

- Normal traffic

- Peak-hour congestion

- Emergency-vehicle movement

The architecture must provide low processing latency, efficient memory access, high I/O throughput and reliable real-time response.

1.2 System Inputs and Outputs

| Inputs | Outputs |
| --- | --- |
| Traffic camera data | Traffic density |
| Vehicle count | Signal timing |
| Vehicle speed | Congestion alert |
| Road occupancy | Emergency priority |
| Emergency-vehicle signal | Traffic status |
| Sensor information | Control commands |

1.3 Architectural Challenges

The major challenges are:

- Large volume of camera data

- Real-time processing requirements

- Frequent memory accesses

- Sensor communication overhead

- Emergency response latency

- Power consumption

- Scalability

2. PROPOSED SYSTEM ARCHITECTURE

The proposed ITMS consists of five major layers:

Traffic Sensors and Cameras
↓
I/O Controller and DMA
↓
Multicore Processor
↓
Cache and Main Memory
↓
Traffic Decision and Signal Controller

The processor analyses traffic information, while DMA transfers large sensor or camera data blocks directly between I/O devices and memory.

[INSERT SYSTEM ARCHITECTURE DIAGRAM HERE]

Major Components

| Component | Function |
| --- | --- |
| Traffic Camera | Captures road images |
| Vehicle Sensor | Measures vehicle count/speed |
| I/O Controller | Manages peripheral communication |
| DMA | Transfers data without continuous CPU involvement |
| Multicore CPU | Processes traffic information |
| L1/L2/L3 Cache | Provides fast data access |
| DRAM | Stores active traffic data |
| Signal Controller | Controls traffic lights |

3. PROCESSOR ARCHITECTURE AND INSTRUCTION SET DESIGN

3.1 Selected Architecture

A 64-bit multicore processor with SIMD/vector processing capability is proposed for the ITMS.

Justification

A multicore processor allows different traffic-processing tasks to execute simultaneously. SIMD operations are useful for processing image pixels and sensor data in parallel.

For example:

- Core 1 → Camera processing

- Core 2 → Vehicle counting

- Core 3 → Traffic-density calculation

- Core 4 → Signal optimisation

This improves response time during high traffic conditions.

3.2 Instruction Set

The processor requires:

- Arithmetic instructions

- Logical instructions

- Load/store instructions

- Branch instructions

- SIMD instructions

- Control instructions

Example

A traffic-density calculation can use arithmetic instructions, while image processing can use SIMD operations to process multiple pixel values simultaneously.

3.3 Instruction Formats

| Format | Application |
| --- | --- |
| Register | Fast calculations |
| Immediate | Threshold values |
| Load/Store | Traffic-data access |
| Branch | Traffic-condition decisions |

3.4 Addressing Modes

Register Addressing

Used when operands are already stored in registers.

Immediate Addressing

Used for constants such as traffic-density thresholds.

Base/Displacement Addressing

Useful for accessing sensor arrays and camera buffers.

PC-Relative Addressing

Used for branch and control instructions.

These addressing modes reduce unnecessary memory operations and support efficient real-time processing.

4. PROCESSOR PERFORMANCE EVALUATION

Processor performance is evaluated using instruction count, CPI, clock cycles, execution time and utilisation.

4.1 Clock Cycle Calculation

CPU Cycles=Instruction Count×CPI

Assume:

- Instruction Count = 20 million

- CPI = 1.5

CPU Cycles=20M×1.5
=30M cycles

4.2 Execution Time

Execution Time=Instruction Count×CPIClock Rate

For a 2.5 GHz processor:

Execution Time=30M2.5G
=0.012s

Therefore, execution time is approximately 12 ms.

4.3 Processor Utilisation

Processor Utilisation=Busy TimeTotal Time×100

For example, if the CPU is active for 700 ms during a 1-second interval:

Utilisation=70%

Lower unnecessary CPU activity is desirable for energy-efficient operation.

5. MEMORY ORGANISATION AND CACHE OPTIMISATION

The proposed memory hierarchy is:

Registers → L1 → L2 → L3 → DRAM → Storage

Frequently accessed traffic information is stored in faster cache levels.

5.1 Proposed Cache

| Cache | Proposed Size | Purpose |
| --- | --- | --- |
| L1 | 64 KB/core | Frequently used instructions/data |
| L2 | 512 KB/core | Working traffic data |
| L3 | 8 MB | Shared traffic information |
| DRAM | 8 GB | Main memory |

5.2 Cache Hit and Miss Rate

Hit Rate=Cache HitsTotal Accesses×100

If 95,000 out of 100,000 accesses are hits:

Hit Rate=95%
Miss Rate=5%

A higher cache hit rate reduces DRAM accesses and improves performance.

6. AMAT ANALYSIS

Average Memory Access Time is calculated using:

AMAT=Hit Time+Miss Rate×Miss Penalty

Assume:

- Hit time = 1 ns

- Miss rate = 5%

- Miss penalty = 50 ns

AMAT=1+0.05×50
AMAT=3.5ns

Therefore, the effective average memory access time is approximately 3.5 ns.

7. CACHE CONFIGURATION COMPARISON

| Configuration | Hit Rate | AMAT | Relative Performance |
| --- | --- | --- | --- |
| L1 only | 88% | 7.0 ns | Low |
| L1 + L2 | 93% | 4.5 ns | High |
| L1 + L2 + L3 | 96% | 3.2 ns | Very High |

The three-level cache provides better performance because it reduces accesses to slower DRAM.

8. MEMORY TECHNOLOGY COMPARISON

| Technology | Characteristic | ITMS Application |
| --- | --- | --- |
| SRAM | Very low latency | Cache |
| DRAM | High capacity | Main memory |
| NAND Flash | Non-volatile | Storage |
| MRAM | Fast, non-volatile | Embedded systems |
| RRAM | High-density potential | Future systems |

SRAM is preferred for cache because of its speed, while DRAM provides economical high-capacity main memory.

9. I/O COMMUNICATION DESIGN

Traffic cameras and sensors continuously generate data. If the CPU handles every transfer, processor utilisation increases.

Therefore, the proposed architecture uses DMA-based I/O.

Data Path

Sensor → I/O Controller → DMA → DRAM → CPU

DMA transfers large blocks of sensor data directly to memory, allowing the CPU to concentrate on traffic processing.

Advantages

- Lower CPU overhead

- Higher data-transfer efficiency

- Better throughput

- Faster real-time response

10. INTERRUPT HANDLING

Interrupts allow devices to notify the processor when immediate attention is required.

Examples

- Emergency vehicle detected

- Critical congestion detected

- Sensor failure

- Camera malfunction

A vectored hardware interrupt is suitable because the processor can directly locate the corresponding interrupt service routine.

Emergency interrupts should receive higher priority than routine sensor updates.

11. MEMORY-MAPPED AND I/O-MAPPED I/O

| Feature | Memory-Mapped I/O | I/O-Mapped I/O |
| --- | --- | --- |
| Address space | Shared with memory | Separate |
| Instructions | Normal load/store | Special I/O instructions |
| Programming | Simpler | More specialised |
| ITMS suitability | High | Moderate |

Memory-mapped I/O is recommended because traffic controllers and sensor registers can be accessed using normal load/store instructions.

12. ADVANCED BUS INTERFACES

| Interface | ITMS Application |
| --- | --- |
| PCIe | High-speed cameras/accelerators |
| USB | Peripheral sensors |
| NVMe | High-speed traffic-data storage |
| Thunderbolt | High-speed external communication |

PCIe is particularly suitable for high-bandwidth traffic cameras and AI accelerators because of its high throughput.

13. TRAFFIC WORKLOAD SCENARIOS

Scenario 1 – Normal Traffic

Moderate traffic density with normal sensor activity.

Expected: Normal signal timing and moderate processor utilisation.

Scenario 2 – Peak-Hour Traffic

High vehicle density and increased camera-processing workload.

Expected: Higher CPU utilisation, more memory accesses and greater cache activity.

Scenario 3 – Emergency Vehicle

Emergency vehicle is detected.

Expected: High-priority interrupt and rapid signal-control response.

14. PERFORMANCE TESTING AND VALIDATION

The system is evaluated using:

- Instruction count

- CPI

- Clock cycles

- Execution time

- Processor utilisation

- Cache hit rate

- Cache miss rate

- AMAT

- Bandwidth

- Throughput

Representative Workload Results

| Metric | Normal | Peak Hour | Emergency |
| --- | --- | --- | --- |
| Instruction Count | 15 M | 25 M | 18 M |
| CPI | 1.4 | 1.7 | 1.3 |
| Execution Time | 8.4 ms | 17.0 ms | 9.36 ms |
| CPU Utilisation | 42% | 78% | 65% |
| Cache Hit Rate | 95% | 91% | 96% |

These values can be replaced with actual simulation measurements.

15. CACHE PERFORMANCE ANALYSIS

The peak-hour workload produces greater memory activity than normal traffic. Therefore, cache efficiency becomes important.

The proposed three-level cache maintains a high hit rate and reduces the number of expensive DRAM accesses.

The emergency workload is given priority so that important control information can be processed with low latency.

[INSERT CACHE HIT/MISS GRAPH HERE]

16. PERFORMANCE COMPARISON OF ARCHITECTURES

| Architecture | CPI | Cache Hit Rate | Execution Time | Cost | Scalability |
| --- | --- | --- | --- | --- | --- |
| Single Core | 2.1 | 88% | High | Low | Low |
| Dual Core | 1.7 | 92% | Medium | Medium | Medium |
| Multicore + L3 | 1.3 | 96% | Low | Higher | High |

The multicore architecture provides the best performance for demanding traffic workloads.

17. PERFORMANCE, POWER AND COST TRADE-OFF

A high-performance architecture generally requires additional hardware and power.

| Design Choice | Advantage | Trade-off |
| --- | --- | --- |
| Multicore CPU | Parallel processing | Higher cost |
| Large Cache | Fewer memory accesses | More hardware |
| DMA | Lower CPU overhead | Additional controller |
| SIMD | Faster image processing | Specialised hardware |
| PCIe | High bandwidth | Higher cost |

The proposed architecture accepts moderate hardware complexity to achieve better real-time performance.

18. IMPLEMENTATION / SIMULATION

The system can be implemented or simulated using a suitable architecture simulation tool such as Logisim Evolution, a processor simulator, or a Python-based performance model.

The simulation should demonstrate:

- Processor operation

- Cache behaviour

- Memory access

- Sensor input

- DMA transfer

- Interrupt handling

- Traffic workload processing

[INSERT ACTUAL SIMULATION SCREENSHOT]

[INSERT ACTUAL CACHE OUTPUT]

[INSERT ACTUAL TRAFFIC SCENARIO OUTPUT]

19. TEST CASES

| Test Case | Input Condition | Expected Result |
| --- | --- | --- |
| TC01 | Normal traffic | Normal signal operation |
| TC02 | High vehicle density | Congestion response |
| TC03 | Emergency vehicle | Priority signal |
| TC04 | Sensor failure | Interrupt/alert |
| TC05 | High memory workload | Cache utilisation increases |
| TC06 | Large sensor transfer | DMA activated |

20. ENGINEERING RECOMMENDATION

Based on the performance analysis, the recommended architecture is:

64-bit Multicore Processor

- SIMD Processing

- L1/L2/L3 Cache

- DRAM

- DMA-Based I/O

- Vectored Interrupts

- PCIe High-Speed Interface

This configuration provides a suitable balance between performance, latency, scalability, reliability and energy efficiency.

The three-level cache reduces memory latency, multicore processing handles parallel workloads and DMA reduces CPU overhead.

21. SYSTEM OPTIMISATION

The proposed system can be optimised by:

- Increasing cache efficiency.

- Using SIMD for image processing.

- Assigning emergency tasks higher priority.

- Using DMA for large data transfers.

- Parallelising independent traffic calculations.

- Reducing unnecessary DRAM accesses.

- Using low-power processing during normal traffic.

These techniques improve both performance and energy efficiency.

22. SDG RELEVANCE

SDG 9 – Industry, Innovation and Infrastructure

The project applies advanced processor, memory and communication technologies to create intelligent transportation infrastructure.

SDG 11 – Sustainable Cities and Communities

Real-time traffic management can reduce congestion and improve transportation efficiency and emergency response.

SDG 13 – Climate Action

Reducing vehicle waiting and unnecessary idling can reduce fuel consumption and associated emissions.

23. RESULTS AND OBSERVATIONS

The analysis indicates that the proposed multicore architecture performs better under demanding traffic conditions than a single-core architecture. The three-level cache improves memory performance by increasing the probability of finding frequently used data close to the processor.

DMA reduces processor involvement in large sensor transfers, while priority interrupts improve emergency-vehicle response.

The main trade-off is increased hardware complexity and cost. However, for a smart-city traffic system, the improvement in real-time performance and scalability justifies the additional architectural resources.

24. LIMITATIONS

Limitations of ITMS

- High Implementation Cost

  - Deployment of traffic cameras, sensors, and monitoring devices requires significant investment.

  - Maintenance and calibration of hardware add recurring expenses.

- Data Reliability & Accuracy

  - Sensors may malfunction due to weather conditions (rain, fog, dust).

  - Camera-based systems can face issues like poor visibility at night or obstruction by large vehicles.

- Integration Challenges

  - Existing traffic infrastructure may not easily support dynamic signal control.

  - Compatibility with legacy systems can slow adoption.

- Cybersecurity Risks

  - Real-time data transmission makes the system vulnerable to hacking or manipulation.

  - Ensuring secure communication between devices and central servers is critical.

- Scalability Issues

  - Large metropolitan areas require massive sensor networks, which may be difficult to scale efficiently.

  - Rural or semi-urban regions may lack infrastructure to support ITMS.

- Dependence on Connectivity

  - Continuous internet or network connectivity is required for real-time decision-making.

  - Any disruption can cause system failure or revert to fixed-timing signals.

- Privacy Concerns

  - Use of cameras raises concerns about surveillance and misuse of personal data.

  - Strict policies are needed to protect citizen privacy.

- Emergency Handling Limitations

  - While ITMS can prioritize emergency vehicles, sensor errors or congestion beyond system capacity may still cause delays.

- Environmental & Physical Constraints

  - Road construction, accidents, or natural disasters can disrupt sensor data flow.

  - Extreme weather may damage equipment or reduce effectiveness.

25. FUTURE ENHANCEMENTS

Future versions can incorporate:

- AI-Powered Predictive Modeling

  - Use machine learning to forecast traffic congestion before it occurs.

  - Predict peak hours, accident-prone zones, and reroute vehicles proactively.

- Integration with Smart City IoT

  - Connect ITMS with smart streetlights, parking systems, and public transport schedules.

  - Enable holistic urban mobility management.

- Autonomous Vehicle Support

  - Provide real-time traffic signal data directly to self-driving cars.

  - Coordinate vehicle-to-infrastructure (V2I) communication for safer navigation.

- Emergency Vehicle Priority System

  - Automatically detect ambulances, fire trucks, and police vehicles.

  - Create green corridors to minimize response time.

- Adaptive Eco-Friendly Signals

  - Optimize traffic flow to reduce idle time and fuel consumption.

  - Integrate with electric vehicle charging stations for smart routing.

- Crowdsourced Traffic Data

  - Allow citizens to report accidents, roadblocks, or unusual congestion via mobile apps.

  - Combine sensor data with human input for richer insights.

- Augmented Reality (AR) Navigation

  - Provide drivers with AR-based guidance through smartphones or car dashboards.

  - Highlight optimal routes and traffic alerts visually.

- Blockchain-Based Data Security

  - Use blockchain to secure traffic data and prevent manipulation.

  - Ensure transparency and trust in system decisions.

- Scalable Cloud Architecture

  - Deploy ITMS on cloud platforms for real-time processing and easy scalability.

  - Support multiple cities with centralized monitoring.

- Environmental Monitoring Integration

- Track air quality and noise pollution alongside traffic density.

- Adjust traffic flow to minimize environmental impact.

26. CONCLUSION

The Intelligent Traffic Management System highlights how computer architecture can directly improve everyday urban life. By harnessing the power of multicore processors, the system can handle multiple streams of traffic data simultaneously, ensuring that decisions are made quickly and reliably. The inclusion of SIMD operations accelerates image processing tasks, such as vehicle recognition from traffic cameras, making the system responsive even under heavy workloads.

A carefully designed hierarchical cache (L1, L2, L3) minimizes delays in accessing frequently used data, while DMA-based I/O ensures that sensor inputs are transferred efficiently without overburdening the CPU. Together, these architectural choices reduce latency and improve throughput, which are critical for real-time traffic control.

Performance metrics such as CPI, instruction count, execution time, processor utilization, cache hit rate, and AMAT demonstrate that the proposed design achieves a balance between speed, scalability, and energy efficiency. This means the ITMS is not only capable of handling current traffic demands but is also adaptable to future smart-city expansions.

In essence, the recommended architecture — multicore processor with three-level cache, DMA-driven I/O, and priority interrupt handling — provides a robust foundation for intelligent traffic management. It ensures that signals respond dynamically to real-world conditions, reduces congestion, and supports emergency services more effectively. Beyond technical efficiency, this design represents a step toward safer, greener, and smarter cities, where technology directly enhances the quality of urban living.

27. INDIVIDUAL CONTRIBUTION

| Team Member | Contribution |
| --- | --- |
| Tanisha | Problem formulation and processor architecture |
| Gopika | Instruction set and performance calculations |
| B.Lakshitha sankar Reddy | Memory hierarchy and cache analysis |

28. INDIVIDUAL REFLECTION

This project helped me understand how computer architecture can be applied to a real-time traffic-management problem. I learned that processor performance depends not only on clock frequency but also on instruction count, CPI, memory access and workload characteristics. The analysis of L1, L2 and L3 cache helped me understand how frequently accessed data can be processed faster by reducing main-memory accesses.

I also understood the importance of DMA and interrupts in real-time I/O systems. DMA reduces unnecessary CPU involvement, while priority interrupts allow emergency events to receive immediate attention. Comparing single-core and multicore architectures helped me understand the trade-off between performance, cost, power and scalability.

The project also showed how computer architecture contributes to sustainable smart-city development. Efficient traffic processing can support better traffic flow, reduce unnecessary vehicle waiting and contribute to lower energy consumption and emissions. Overall, the assignment improved my understanding of CPU performance, memory hierarchy and I/O communication while connecting these concepts with SDG 9, SDG 11 and SDG 13.

29. REFERENCES

- William Stallings, Computer Organization and Architecture, Pearson.

- David A. Patterson and John L. Hennessy, Computer Organization and Design.

- M. Morris Mano, Computer System Architecture.

- Logisim Evolution Documentation.

- PCI-SIG, PCI Express Technical Specifications.

- NVM Express Technical Specifications.

- USB Implementers Forum, USB Specifications.